# REQUEST FOR QUOTATION (RFQ)

RFQ No.: EEPL/PROC/2025-26/1021
 Date: 15-01-2026

To:
 M/s ACME Process Systems Pvt. Ltd.
 Pune, Maharashtra, India

## Subject: RFQ for Turnkey Supply of Solvent Recovery / Process Plant

Dear Sir/Madam,

We request you to submit your techno-commercial quotation for the design, engineering, manufacturing, supply, and commissioning of a turnkey process plant as per the details mentioned below.

## 1. Scope Overview

The project involves complete turnkey execution of a solvent/process plant including:

- Storage tanks

- Process tanks

- Day tanks & receiver tanks

- Piping & pump systems

- Automation (PLC/SCADA)

- Structural supports & platforms

## 2. Major Equipment Scope

### 2.1 Tank Farm & Storage Systems

- Storage Tanks: Up to 40 KL capacity (multiple units)

- Day Tanks: 1–2 KL range (multiple units)

- Receiver Tanks: 300 L – 500 L

### 2.2 Process Tanks

- Typical Capacity: ~2 KL

- Design Code: ASME Section VIII Div.1 & ASME BPE

- Operating Temperature: Up to ~150°C

- Design Temperature: Up to ~195°C

- Operating Pressure: FV to 3 kg/cm²

- MOC (Wetted): SS316L

- Features:

  - Limpet coil for heating/cooling

  - Spray balls for CIP

  - Baffles and internal fittings

### 2.3 Example Tank Specification (Reference Unit)

- Working Capacity: ~2–2.5 KL

- Diameter: ~1300–1400 mm

- Height: ~1700–1950 mm

- Orientation: Vertical cylindrical

- Dish Ends: Torispherical

- Thickness: ~5–6 mm

### 2.4 Utility & Auxiliary Systems

- Pumps (as per P&ID)

- Solvent pre-filters and final filters (SS316 construction)

- Pipe racks and interconnecting piping

- Insulation systems (rockwool + cladding)

### 2.5 Automation & Electrical Scope

- PLC panel & programming

- SCADA system

- HMI stations (FLP areas)

- MCC panels

- Instrumentation & control wiring

## 3. Scope of Supply

Vendor to include:

- Design & detailed engineering

- Fabrication of all vessels and tanks

- Supply of pumps, valves, and instruments

- Process piping fabrication & installation

- Automation system (PLC/SCADA)

- Erection, commissioning & trial run

- FAT & SAT execution

## 4. Scope Clarifications

### Vendor Scope

- Equipment supply

- Piping & installation

- Automation & control system

- Trial run & performance testing

### Client Scope

- Civil works

- Utilities & consumables

- HVAC & ventilation

- Material handling at site

## 5. Design & Standards

- ASME Section VIII Div. 1

- ASME BPE (for hygienic/process tanks)

- API 650 (for storage tanks where applicable)

## 6. Testing & Quality Requirements

- Hydrotesting as per code

- Radiography (spot/full as applicable)

- Material Test Certificates (MTCs)

- FAT & SAT

## 7. Commercial Requirements

Kindly provide:

- Complete package price (itemized if possible)

- Delivery & project timeline

- Payment terms

- GST and applicable taxes

- Warranty terms

- Offer validity

## 8. Documentation

- P&ID

- GA drawings

- Layout drawings

- Datasheets

- Operation & maintenance manuals

## 9. Additional Notes

- System to be designed for continuous operation with solvent handling

- Flameproof area compliance to be considered

- All equipment to be compatible with process fluids (client to confirm)

We request you to submit your detailed techno-commercial offer at the earliest.

Thanks & Regards,
 Procurement Team
 EEPL