# REQUEST FOR QUOTATION (RFQ)

RFQ No.: EEPL/PROC/2025-26/1187
 Date: 10-01-2026

To:
 M/s ACME Process Systems Pvt. Ltd.
 Pune, Maharashtra, India

## Subject: RFQ for Supply of Distillation System (Columns, Condensers & Flash Vessels)

Dear Sir/Madam,

We request you to submit your techno-commercial quotation for the design, manufacturing, and supply of a distillation system package comprising columns, condensers, and flash vessels as per the details below.

## 1. Scope Summary

The scope includes supply of:

- Distillation Columns

- Main Condensers

- Vent Condensers

- Flash Vessels (multiple sizes)

All equipment shall be designed as per applicable codes and integrated for process compatibility.

## 2. Flash Vessels

### Type 1 Flash Vessels

- Tags: Multiple (FV series)

- Quantity: ~10 Nos

- Working Volume: ~6 KL

- Design Code: ASME Section VIII Div.1

Process Conditions:

- Operating Temp: ~180–280°C

- Design Temp: Up to 300°C

- Operating Pressure: 1.5 bar / Vacuum

- Design Pressure: 3.5 bar / Full Vacuum / up to 20 bar

Construction:

- Vertical cylindrical with torispherical ends

- Limpet coil for heating

- MOC: SS316 (wetted parts)

### Type 2 Flash Vessels

- Quantity: ~5 Nos

- Working Volume: ~8 KL

- Similar design and operating conditions as above

## 3. Main Condensers

- Type: Shell & Tube Heat Exchangers

- Quantity: ~15 Nos

- Heat Transfer Area: ~20 m²

Design Standards:

- ASME Section VIII Div.1

- TEMA Class C

Key Parameters:

- Shell Side Temp: ~150–165°C

- Tube Side Temp: ~35–40°C

- Tube Material: SS316

## 4. Vent Condensers

- Quantity: ~15 Nos

- Heat Transfer Area: ~5 m²

Configuration:

- Shell & tube type

- Vertical orientation

Operating Conditions:

- Vacuum service (down to ~1 torr)

## 5. Distillation Columns

- Tags: Multiple DC series

- Quantity: ~15 Nos

Mechanical Details:

- Diameter: ~800 mm

- Height: ~23,800 mm

- Orientation: Vertical

Process Conditions:

- Operating Temp: ~170°C

- Design Temp: ~250°C

- Operating Pressure: Vacuum / low pressure

Notes:

- Packing & internals in client scope

- Only support rings to be included

## 6. Materials of Construction

- Wetted parts: SS316

- Structural/support components: Carbon Steel with SS overlay (where required)

- Gaskets: PTFE

## 7. Scope of Supply

Vendor to include:

- Design & detailed engineering

- Fabrication of all vessels and heat exchangers

- Supply of all nozzles, flanges, and supports

- Material Test Certificates (MTCs)

- Shop testing (hydrotest, NDT)

## 8. Exclusions

- Column internals (packing, trays)

- Insulation

- Installation & commissioning

- Third-party inspection

## 9. Testing & Quality

- Hydrotesting as per ASME

- Radiography: 10% / 100% as applicable

- Surface finish: Pickling & passivation

## 10. Commercial Requirements

Kindly provide:

- Item-wise pricing

- Delivery timeline (expected ~12–20 weeks)

- Payment terms (advance + dispatch)

- GST and applicable taxes

- Warranty terms

## 11. Documentation

- GA drawings

- Fabrication drawings

- Datasheets

- Deviation list

We request you to submit your quotation at the earliest.

Thanks & Regards,
 Procurement Team
 EEPL