### REQUEST FOR QUOTATION (RFQ)

RFQ No.: EEPL/PROC/2025-26/711
 Date: 01-08-2025

To:
 M/s ACME Process Systems Pvt. Ltd.
 Pune, Maharashtra, India

### Subject: RFQ for Design, Manufacturing & Supply of Process Column (Tag: C-1301)

Dear Sir/Madam,

We request you to submit your techno-commercial quotation for the design, fabrication, and supply of a process column as per the specifications outlined below.

### 1. Equipment Details

- Item: Process Column

- Tag No.: C-1301

- Quantity: 1 No.

- Type: Vertical Column

### 2. Process Parameters

- Process Fluid: To be confirmed

- Flow Rate: To be confirmed

- Operating Temperature: ~100°C

- Design Temperature: Up to 180°C

- Operating Pressure: Atmospheric / Full Vacuum

- Design Pressure: FV / 3.5 kg/cm²(g)

### 3. Mechanical & Design Requirements

- Design Code: ASME Section VIII Div. 1

- Material of Construction (Preferred): SS316

- Corrosion Allowance:

  - SS: Nil

  - CS: 1.5 mm

- Radiography: As per code requirements

- Joint Efficiency: As per ASME standards

### 4. Column Configuration

- Orientation: Vertical

- Inside Diameter: ~550 mm

- Height (TL-TL): ~16,500 mm

- Type: Cylindrical with torispherical top and flanged bottom

- Working Capacity: ~3.2 KL (to be confirmed)

### 5. Internals & Accessories

(To be quoted separately if applicable)

- Packing support grid

- Structured/random packing

- Liquid distributor & collector

- Bed limiter

Note: Vendor may recommend suitable internals based on process data.

### 6. Scope of Supply

Please include:

- Column shell with all nozzles and supports

- Internal supports (if in vendor scope)

- Flanges, gaskets, and fasteners

- Lifting lugs, earthing boss, and supports

- Material test certificates (MTCs)

### 7. Exclusions (Unless Specified Separately)

- Column internals (packing, distributor, collector)

- Insulation

- Installation & commissioning

- Third-party inspection

- Field instrumentation

### 8. Surface Finish & Testing

- Internal/External: Pickling & passivation

- Hydrotest as per design code

- NDT as per ASME requirements

### 9. Commercial Requirements

Kindly provide:

- Detailed price breakup

- Delivery schedule

- Payment terms

- GST and applicable taxes

- Warranty terms

- Offer validity

### 10. Documentation Requirements

- GA drawing

- Fabrication drawings

- Datasheet compliance sheet

- Deviation list (if any)

We request you to submit your quotation at the earliest.

Please contact us for any clarifications required during estimation.

Thanks & Regards,
 Procurement Team
 EEPL