### REQUEST FOR QUOTATION (RFQ)

RFQ No.: EEPL/PROC/2025-26/925
 Date: 25-08-2025

To:
 M/s ACME Process Systems Pvt. Ltd.
 Pune, Maharashtra, India

#### Subject: RFQ for Supply of Reactor with Agitator (10 KL Capacity)

Dear Sir/Madam,

We request you to submit your techno-commercial quotation for the design, manufacturing, and supply of reactors with agitators as per the details mentioned below.

### 1. Equipment Details

Item: Reactor with Agitator
 Quantity: 2 Nos
 Type: Vertical Cylindrical Reactor

### 2. Process Requirements

- Working Capacity: 10 KL

- Process Fluid: To be handled (details to be shared during detailed engineering)

- Operating Temperature: ~40°C

- Design Temperature Range: -30°C to 180°C

- Operating Pressure: Atmospheric / Full Vacuum

- Design Pressure: Full Vacuum / 3.5 kg/cm²(g)

- Fluid Density: ~1500 kg/m³

- Viscosity: ~4000 cP

### 3. Mechanical & Design Requirements

- Material of Construction (Preferred):

  - Wetted Parts: SS316

  - Alternate Option: SS304 (quote separately)

- Design Code: ASME Section VIII Div. 1

- Corrosion Allowance: 1 mm

- Joint Efficiency: As per ASME standards

- Radiography: As per applicable code requirements

### 4. Vessel Configuration

- Vertical orientation

- Cylindrical shell with torispherical ends

- Approx. dimensions to suit 10 KL working capacity

- Required nozzles, manhole, supports, and internals to be included

### 5. Agitator Requirements

- Type: Top entry agitator

- Impeller: Suitable for high viscosity (~4000 cP)

- Motor: Flameproof, suitable for VFD operation

- Seal: Mechanical seal (single or double, recommend suitable option)

- Gearbox: Inline/helical type preferred

### 6. Scope of Supply

Please include:

- Reactor vessel with all internals

- Agitator assembly with motor & gearbox

- Mechanical seal system

- All nozzles, supports, and accessories

- Material test certificates

### 7. Exclusions (Unless Specified Separately)

- Electrical control panel

- Installation & commissioning

- Field wiring

- Insulation

### 8. Commercial Requirements

Kindly provide:

- Item-wise pricing (SS316 and SS304 options separately)

- Delivery schedule

- Payment terms

- GST and other applicable taxes

- Warranty terms

- Validity of offer

### 9. Additional Requirements

- General arrangement (GA) drawing

- Datasheets for review

- List of deviations (if any)

We request you to submit your quotation at the earliest.

For any clarifications, please feel free to contact us.

Thanks & Regards,
 Procurement Team
 EEPL